SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 3 – CODING CHALLENGE QUESTIONS

ANNEXURE A

| Course Code: | CSA12 |
| --- | --- |
| Course Title: | Computer Architecture |
| CO Assessed: | CO1 |
| Assessment: | Assessment Tool 3 – Coding Challenge Questions |
| Weightage: | 40% |
| Total Marks: | 25 |
| CO1 Statement: | Analyze the functional units of a computer system including CPU, memory, bus structures, and I/O components by examining instruction execution cycles, addressing modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4) |
| Student Name: | Kanishk C |
| Reg. No.: | 192511278 |
| Department: | Computer Science and Engineering |
| Date: | 26.07.2026 |

Code of Conduct:

I, Kanishk C (Reg. No.: 192511278), certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Kanishk C

# Rubrics for Calculation

| Criteria | Weight | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly understands problem constraints, edge cases, and requirements | Understands problem with minor gaps in constraints | Partial understanding; misses key constraints | Misinterprets problem or constraints |
| Algorithm | 25% | Selects optimal algorithm with best time/space complexity | Uses efficient algorithm with minor scope for improvement | Uses basic/inefficient approach | No clear algorithmic approach |
| Code Implementation | 20% | Writes correct, efficient, clean code that passes all constraints | Code works with minor errors or inefficiencies | Partially correct code; fails for some cases | Code does not execute or gives incorrect results |
| Competitive Performance | 20% | Solves within time limits and handles large inputs effectively | Solves within acceptable time with minor delays | Struggles with time limits or larger inputs | Unable to solve within time constraints |
| Test Case Handling | 15% | Handles all test cases including edge and hidden cases | Handles most test cases | Misses important edge cases | Fails on multiple test cases |

# Question 1 – CPU Simulator

Design and implement a CPU simulator that executes a set of basic instructions (LOAD, STORE, ADD, SUB, MOV, JMP) by simulating the instruction fetch, decode, execute, memory access, and write-back stages. Display the register and memory contents after each instruction execution and calculate the total execution cycles.

## Approach

The simulator is written in Python and models a small CPU with four general purpose registers (R0–R3) and a 32-word memory array. Every instruction is passed through the five classic pipeline stages one after another – Fetch, Decode, Execute, Memory and Write-Back – and each stage costs one clock cycle. For arithmetic instructions the memory stage does no real work (it is a bubble), while LOAD and STORE genuinely read or write memory in that stage. After every instruction the simulator prints the full register file and the first few memory locations so the state change is visible step by step, and at the end it reports the total number of cycles the whole program took.

A small test program was used to check the simulator: it loads two values into registers, adds them, stores the result to memory, loads it back into a third register, and finally subtracts a register from it. This exercises LOAD, STORE, ADD, SUB and MOV together.

Source Code:

"""
Question 1 - CPU Simulator
Supports: LOAD, STORE, ADD, SUB, MOV, JMP
Simulates Fetch -> Decode -> Execute -> Memory Access -> Write-Back
Prints registers and memory after every instruction and totals the cycles.
"""
 
# Each instruction is charged a fixed number of cycles per stage.
# This mirrors the classic 5-stage pipeline cost model used in the
# CO1 lecture (1 cycle per stage, memory stage only "does work" for
# LOAD/STORE, otherwise it is a bubble).
CYCLES_PER_STAGE = 1
STAGES = ["FETCH", "DECODE", "EXECUTE", "MEMORY", "WRITE-BACK"]
 
 
class CPU:
    def __init__(self, mem_size=32):
        self.registers = {f"R{i}": 0 for i in range(4)}   # R0..R3
        self.memory = [0] * mem_size
        self.pc = 0
        self.total_cycles = 0
        self.history = []
 
    def show_state(self, instr_text):
        print(f"\nAfter executing: {instr_text}")
        print("Registers :", self.registers)
        print("Memory[0:8]:", self.memory[0:8])
 
    def run(self, program):
        self.program = program
        while self.pc < len(program):
            raw = program[self.pc]
            parts = raw.replace(",", " ").split()
            op = parts[0].upper()
 
            # ---- FETCH ----
            self.total_cycles += CYCLES_PER_STAGE
 
            # ---- DECODE ----
            self.total_cycles += CYCLES_PER_STAGE
            args = parts[1:]
 
            # ---- EXECUTE ----
            self.total_cycles += CYCLES_PER_STAGE
            next_pc = self.pc + 1
 
            if op == "LOAD":           # LOAD R0, 5      -> R0 = MEM[5]
                dest, addr = args[0], int(args[1])
                # ---- MEMORY (read) ----
                self.total_cycles += CYCLES_PER_STAGE
                value = self.memory[addr]
                # ---- WRITE-BACK ----
                self.total_cycles += CYCLES_PER_STAGE
                self.registers[dest] = value
 
            elif op == "STORE":        # STORE R0, 5     -> MEM[5] = R0
                src, addr = args[0], int(args[1])
                # ---- MEMORY (write) ----
                self.total_cycles += CYCLES_PER_STAGE
                self.memory[addr] = self.registers[src]
                # ---- WRITE-BACK (no register touched) ----
                self.total_cycles += CYCLES_PER_STAGE
 
            elif op == "ADD":          # ADD R0, R1, R2  -> R0 = R1 + R2
                dest, s1, s2 = args
                # MEMORY stage is a bubble for ALU ops
                self.total_cycles += CYCLES_PER_STAGE
                result = self.registers[s1] + self.registers[s2]
                self.total_cycles += CYCLES_PER_STAGE
                self.registers[dest] = result
 
            elif op == "SUB":          # SUB R0, R1, R2  -> R0 = R1 - R2
                dest, s1, s2 = args
                self.total_cycles += CYCLES_PER_STAGE
                result = self.registers[s1] - self.registers[s2]
                self.total_cycles += CYCLES_PER_STAGE
                self.registers[dest] = result
 
            elif op == "MOV":          # MOV R0, 10  or  MOV R0, R1
                dest, src = args
                self.total_cycles += CYCLES_PER_STAGE
                value = int(src) if src.lstrip('-').isdigit() else self.registers[src]
                self.total_cycles += CYCLES_PER_STAGE
                self.registers[dest] = value
 
            elif op == "JMP":          # JMP 4  -> jump to instruction index 4
                target = int(args[0])
                self.total_cycles += CYCLES_PER_STAGE   # memory bubble
                self.total_cycles += CYCLES_PER_STAGE   # write-back bubble
                next_pc = target
 
            else:
                raise ValueError(f"Unknown instruction: {op}")
 
            self.show_state(raw)
            self.pc = next_pc
 
        print(f"\nTotal instructions executed : {len(program)}")
        print(f"Total execution cycles       : {self.total_cycles}")
 
 
if __name__ == "__main__":
    program = [
        "MOV R0, 10",
        "MOV R1, 20",
        "ADD R2, R0, R1",
        "STORE R2, 0",
        "LOAD R3, 0",
        "SUB R3, R3, R0",
    ]
    cpu = CPU()
    cpu.run(program)

Sample Output:

After executing: MOV R0, 10
Registers : {'R0': 10, 'R1': 0, 'R2': 0, 'R3': 0}
Memory[0:8]: [0, 0, 0, 0, 0, 0, 0, 0]
 
After executing: MOV R1, 20
Registers : {'R0': 10, 'R1': 20, 'R2': 0, 'R3': 0}
Memory[0:8]: [0, 0, 0, 0, 0, 0, 0, 0]
 
After executing: ADD R2, R0, R1
Registers : {'R0': 10, 'R1': 20, 'R2': 30, 'R3': 0}
Memory[0:8]: [0, 0, 0, 0, 0, 0, 0, 0]
 
After executing: STORE R2, 0
Registers : {'R0': 10, 'R1': 20, 'R2': 30, 'R3': 0}
Memory[0:8]: [30, 0, 0, 0, 0, 0, 0, 0]
 
After executing: LOAD R3, 0
Registers : {'R0': 10, 'R1': 20, 'R2': 30, 'R3': 30}
Memory[0:8]: [30, 0, 0, 0, 0, 0, 0, 0]
 
After executing: SUB R3, R3, R0
Registers : {'R0': 10, 'R1': 20, 'R2': 30, 'R3': 20}
Memory[0:8]: [30, 0, 0, 0, 0, 0, 0, 0]
 
Total instructions executed : 6
Total execution cycles       : 30

## Observation

Each instruction always passes through all five stages in this model, so with five stages costing one cycle each, six instructions come out to exactly 30 cycles (6 × 5), which matches the printed total. STORE and LOAD are the only two instructions whose memory stage actually touches the memory array, and this shows clearly in the trace: memory location 0 stays at 0 until the STORE instruction runs, after which it becomes 30 and the following LOAD correctly reads that same value back into R3.

# Question 2 – Bus Architecture Simulator

Develop a program to simulate single-bus, multi-bus, and hierarchical bus architectures. Implement communication between CPU, Memory, and I/O devices, measure data transfer time, detect bus contention, and compare throughput and latency for each bus structure.

## Approach

A fixed set of eight random transfer requests is generated first, each one a (source, destination, word-count) tuple among CPU, MEMORY, IO1 and IO2, and the same request list is then fed into all three bus models so the comparison at the end is fair.

In the single-bus model every transfer has to wait for the one shared bus to become completely free before it can start, so the requests are simply queued one after another; any request that has to wait because the bus is still busy from the previous transfer is logged as a contention.

In the multi-bus model there are two independent buses – one dedicated to CPU–MEMORY traffic and one dedicated to CPU–I/O traffic – so transfers that use different buses can effectively overlap, which is the main advantage this architecture is meant to demonstrate.

In the hierarchical-bus model there is a fast local bus close to the CPU and a slower, shared system bus that reaches memory and the I/O devices. Small CPU-to-memory transfers are treated as being served from a local cache over the fast local bus, while everything else has to go over the slower, contended system bus.

Source Code:

"""
Question 2 - Bus Architecture Simulator
Simulates single-bus, multi-bus and hierarchical-bus systems.
Measures data transfer time, detects contention, compares
throughput and latency for the three structures.
"""
 
import random
 
random.seed(7)   # keep results repeatable for the report
 
TRANSFER_TIME_PER_WORD = 2   # ns, time the bus takes to move one word
DEVICES = ["CPU", "MEMORY", "IO1", "IO2"]
 
 
def make_requests(n):
    """Create n random (source, destination, words) transfer requests."""
    reqs = []
    for _ in range(n):
        src, dst = random.sample(DEVICES, 2)
        words = random.randint(1, 4)
        reqs.append((src, dst, words))
    return reqs
 
 
def simulate_single_bus(requests):
    """
    Single shared bus: only one transfer can use the bus at a time,
    so every request has to wait for the bus to become free first.
    """
    bus_free_at = 0
    contentions = 0
    log = []
    for i, (src, dst, words) in enumerate(requests):
        start = bus_free_at
        if i > 0 and start > 0:
            # this request had to wait because the previous transfer
            # was still holding the bus -> counts as contention
            contentions += 1
        duration = words * TRANSFER_TIME_PER_WORD
        end = start + duration
        log.append((src, dst, words, start, end))
        bus_free_at = end
    total_time = bus_free_at
    return log, total_time, contentions
 
 
def simulate_multi_bus(requests):
    """
    Multi-bus: one dedicated bus for CPU<->MEMORY traffic and a
    second dedicated bus for CPU<->I/O traffic, so the two kinds of
    transfer can run in parallel unless they land on the same bus.
    """
    mem_bus_free_at = 0
    io_bus_free_at = 0
    log = []
    for src, dst, words in requests:
        duration = words * TRANSFER_TIME_PER_WORD
        if "MEMORY" in (src, dst):
            start = mem_bus_free_at
            end = start + duration
            mem_bus_free_at = end
            bus_used = "MEM-BUS"
        else:
            start = io_bus_free_at
            end = start + duration
            io_bus_free_at = end
            bus_used = "IO-BUS"
        log.append((src, dst, words, start, end, bus_used))
    total_time = max(mem_bus_free_at, io_bus_free_at)
    return log, total_time
 
 
def simulate_hierarchical_bus(requests):
    """
    Hierarchical bus: a fast local bus links CPU to a cache/local
    memory instantly-ish, and a slower system bus links everything
    down to main memory and I/O. Local-only traffic is cheap; system
    bus traffic is expensive and shared, so it is the bottleneck.
    """
    LOCAL_BUS_TIME_PER_WORD = 1     # faster, close to the CPU
    SYSTEM_BUS_TIME_PER_WORD = 3    # slower, shared, has to reach further
 
    system_bus_free_at = 0
    log = []
    for src, dst, words in requests:
        if src == "CPU" and dst == "MEMORY" and words <= 2:
            # small CPU<->memory transfers assumed served from local cache
            duration = words * LOCAL_BUS_TIME_PER_WORD
            start = 0  # local bus is private, never contended in this model
            end = start + duration
            log.append((src, dst, words, start, end, "LOCAL-BUS"))
        else:
            duration = words * SYSTEM_BUS_TIME_PER_WORD
            start = system_bus_free_at
            end = start + duration
            system_bus_free_at = end
            log.append((src, dst, words, start, end, "SYSTEM-BUS"))
    total_time = system_bus_free_at
    return log, total_time
 
 
def report(name, log, total_time, total_words):
    throughput = total_words / total_time if total_time else 0
    avg_latency = sum(e - s for *_ , s, e in [(r[0], r[1], r[2], r[-3], r[-2]) for r in log]) / len(log)
    print(f"\n--- {name} ---")
    for row in log:
        print(" ", row)
    print(f"Total time      : {total_time} ns")
    print(f"Throughput      : {throughput:.3f} words/ns")
    print(f"Average latency : {avg_latency:.3f} ns/request")
 
 
if __name__ == "__main__":
    requests = make_requests(8)
    print("Generated transfer requests (src, dst, words):")
    for r in requests:
        print(" ", r)
    total_words = sum(w for _, _, w in requests)
 
    log1, t1, contentions = simulate_single_bus(requests)
    report("Single-Bus Architecture", log1, t1, total_words)
    print(f"Bus contentions detected : {contentions}")
 
    log2, t2 = simulate_multi_bus(requests)
    report("Multi-Bus Architecture", log2, t2, total_words)
 
    log3, t3 = simulate_hierarchical_bus(requests)
    report("Hierarchical-Bus Architecture", log3, t3, total_words)
 
    print("\n--- Comparison Summary ---")
    print(f"{'Architecture':25}{'Total Time(ns)':18}{'Throughput(words/ns)'}")
    print(f"{'Single Bus':25}{t1:<18}{total_words/t1:.3f}")
    print(f"{'Multi Bus':25}{t2:<18}{total_words/t2:.3f}")
    print(f"{'Hierarchical Bus':25}{t3:<18}{total_words/t3:.3f}")

Sample Output:

Generated transfer requests (src, dst, words):
  ('IO1', 'CPU', 4)
  ('CPU', 'IO2', 1)
  ('IO1', 'IO2', 1)
  ('MEMORY', 'CPU', 1)
  ('IO2', 'MEMORY', 1)
  ('MEMORY', 'CPU', 4)
  ('CPU', 'IO1', 1)
  ('MEMORY', 'IO1', 1)
 
--- Single-Bus Architecture ---
Total time      : 28 ns
Throughput      : 0.500 words/ns
Average latency : 12.000 ns/request
Bus contentions detected : 7
 
--- Multi-Bus Architecture ---
Total time      : 14 ns
Throughput      : 1.000 words/ns
Average latency : 3.500 ns/request
 
--- Hierarchical-Bus Architecture ---
Total time      : 42 ns
Throughput      : 0.333 words/ns
Average latency : 5.250 ns/request
 
--- Comparison Summary ---
Architecture             Total Time(ns)    Throughput(words/ns)
Single Bus               28                0.500
Multi Bus                14                1.000
Hierarchical Bus         42                0.333

## Observation

With this particular request mix the multi-bus system finishes fastest and gives the best throughput, because splitting memory traffic and I/O traffic onto two separate buses lets them run in parallel instead of queueing behind each other. The single bus is slower because every request, regardless of who it is talking to, has to wait its turn on the one shared line. The hierarchical bus actually comes out slowest here, which looks surprising at first, but it makes sense once the traffic pattern is considered: this workload is dominated by MEMORY–CPU and IO–CPU transfers rather than small local CPU-cache accesses, so almost every request is forced onto the slower shared system bus instead of the fast local bus. A hierarchical bus only pays off when most of the traffic is genuinely local; when it is not, it can behave like a single bus with a slower clock. This matches what the CO1 material says about locality of reference being the key factor behind hierarchical-bus performance.

# Question 3 – Benchmarking Tool

Create a benchmarking tool that accepts Instruction Count (IC), Clock Cycle Time, Clock Frequency, and CPI for multiple programs or processors. Compute CPU execution time, MIPS, IPC, and speedup, then compare processor performance using benchmark datasets and generate a detailed performance report.

## Approach

The tool works from the standard performance equations used in the course: CPU execution time = Instruction Count × CPI × Clock Cycle Time, where Clock Cycle Time is simply 1 ÷ Clock Frequency. From the execution time, MIPS is computed as the number of instructions (in millions) divided by the execution time in seconds, and IPC is just the reciprocal of CPI. Speedup for each processor is measured against a chosen baseline processor, so the report shows at a glance how many times faster (or slower) each processor is compared to the baseline on the same program.

A Processor class stores the raw inputs (IC, clock frequency, CPI) and computes all the derived metrics automatically when it is created. The benchmark_report function then prints a formatted comparison table for a whole list of processors and highlights which one is fastest for that program. Two datasets were tried – a smaller and a larger program – both run across the same three example processors (a basic, a pipelined and a superscalar design) to show that the ranking can be checked against more than one workload.

Source Code:

"""
Question 3 - Benchmarking Tool
Accepts Instruction Count (IC), Clock Cycle Time, Clock Frequency and
CPI for several programs/processors, then computes CPU execution
time, MIPS, IPC and speed-up, and prints a performance report.
"""
 
 
def cpu_time(ic, cpi, cycle_time):
    return ic * cpi * cycle_time
 
 
def mips(ic, exec_time_seconds):
    # MIPS = instructions (in millions) per second
    return (ic / 1_000_000) / exec_time_seconds
 
 
def ipc(cpi):
    return 1 / cpi
 
 
class Processor:
    def __init__(self, name, ic, clock_freq_hz, cpi):
        self.name = name
        self.ic = ic
        self.clock_freq_hz = clock_freq_hz
        self.cycle_time = 1 / clock_freq_hz
        self.cpi = cpi
        self.exec_time = cpu_time(ic, cpi, self.cycle_time)
        self.mips = mips(ic, self.exec_time)
        self.ipc = ipc(cpi)
 
 
def benchmark_report(processors, baseline_name):
    baseline = next(p for p in processors if p.name == baseline_name)
 
    header = f"{'Processor':12}{'IC':>12}{'Freq(MHz)':>12}{'CPI':>8}{'ExecTime(s)':>14}{'MIPS':>10}{'IPC':>8}{'Speedup':>10}"
    print(header)
    print("-" * len(header))
    for p in processors:
        speedup = baseline.exec_time / p.exec_time
        print(f"{p.name:12}{p.ic:>12,}{p.clock_freq_hz/1e6:>12.1f}{p.cpi:>8.2f}"
              f"{p.exec_time:>14.6f}{p.mips:>10.2f}{p.ipc:>8.3f}{speedup:>10.3f}")
 
    fastest = min(processors, key=lambda p: p.exec_time)
    print(f"\nFastest processor for this program mix: {fastest.name} "
          f"({fastest.exec_time:.6f} s execution time)")
    return fastest
 
 
if __name__ == "__main__":
    # Sample dataset: three processors running the same program,
    # figures are the kind an instructor would hand out for CO1.
    processors = [
        Processor("P1-Basic",   ic=50_000_000, clock_freq_hz=1_000_000_000, cpi=1.5),
        Processor("P2-Pipelined", ic=50_000_000, clock_freq_hz=1_200_000_000, cpi=1.1),
        Processor("P3-Superscalar", ic=50_000_000, clock_freq_hz=900_000_000, cpi=0.6),
    ]
    benchmark_report(processors, baseline_name="P1-Basic")
 
    print("\nSecond dataset: same three processors, a different (larger) program")
    processors2 = [
        Processor("P1-Basic",   ic=120_000_000, clock_freq_hz=1_000_000_000, cpi=1.6),
        Processor("P2-Pipelined", ic=120_000_000, clock_freq_hz=1_200_000_000, cpi=1.2),
        Processor("P3-Superscalar", ic=120_000_000, clock_freq_hz=900_000_000, cpi=0.7),
    ]
    benchmark_report(processors2, baseline_name="P1-Basic")

Sample Output:

Processor             IC   Freq(MHz)     CPI   ExecTime(s)      MIPS     IPC   Speedup
--------------------------------------------------------------------------------------
P1-Basic      50,000,000      1000.0    1.50      0.075000    666.67   0.667     1.000
P2-Pipelined  50,000,000      1200.0    1.10      0.045833   1090.91   0.909     1.636
P3-Superscalar  50,000,000       900.0    0.60      0.033333   1500.00   1.667     2.250
 
Fastest processor for this program mix: P3-Superscalar (0.033333 s execution time)
 
Second dataset: same three processors, a different (larger) program
Processor             IC   Freq(MHz)     CPI   ExecTime(s)      MIPS     IPC   Speedup
--------------------------------------------------------------------------------------
P1-Basic     120,000,000      1000.0    1.60      0.192000    625.00   0.625     1.000
P2-Pipelined 120,000,000      1200.0    1.20      0.120000   1000.00   0.833     1.600
P3-Superscalar 120,000,000       900.0    0.70      0.093333   1285.71   1.429     2.057
 
Fastest processor for this program mix: P3-Superscalar (0.093333 s execution time)

## Observation

Even though P3-Superscalar runs at the lowest clock frequency of the three (900 MHz versus 1000–1200 MHz for the others), it still finishes fastest in both datasets because its CPI is much lower. This is exactly the point the CO1 material makes about MIPS and raw clock speed being misleading measures on their own – execution time, which folds in IC, CPI and cycle time together, is the metric that actually tells you which processor is faster for a given program. It is also worth noting that MIPS itself is not always reliable across programs with different instruction mixes, which is why the speedup column (based on real execution time) is the more trustworthy comparison here.

# Question 4 – 8085/8086 Addressing Mode Simulator

Implement a simulator for selected 8085/8086 instructions supporting immediate, direct, indirect, register, indexed, and based addressing modes. The simulator should execute user-defined assembly instructions, update registers and flags, and display memory changes after execution.

## Approach

The simulator keeps six registers (A, B, C, D, SI, BX), a 64-word memory array and three flags (Zero, Sign, Carry). Each instruction line is tokenised and dispatched to the addressing mode it represents:

| Instruction | Addressing Mode | What it does |
| --- | --- | --- |
| MVI R,#data | Immediate | Loads a constant written directly in the instruction into a register |
| LDA / STA addr | Direct | Reads/writes a memory address that is written directly in the instruction |
| LDAX (R) | Indirect | The register holds the memory address to be accessed, not the data itself |
| MOV Rd,Rs | Register | Copies data straight from one register to another |
| LDX [SI+off] | Indexed | Address = base register SI + a constant offset written in the instruction |
| LDB [BX+disp] | Based | Address = base register BX + a displacement, same idea as indexed but conventionally used with BX |

ADD and SUB update the Zero, Sign and Carry flags based on the arithmetic result, exactly the way the 8085 ALU would. A short test program exercises every addressing mode once and finishes with HLT.

Source Code:

"""
Question 4 - 8085/8086 Instruction Simulator
Supports immediate, direct, indirect, register, indexed and based
addressing modes. Executes user-defined assembly instructions,
updates registers and flags, and shows memory changes.
 
Instruction formats used in this simulator:
  MVI  R, #data          -> Immediate addressing
  LDA  addr               -> Direct addressing
  LDAX (R)                -> Indirect addressing (register holds address)
  MOV  Rd, Rs              -> Register addressing
  LDX  [SI+offset]        -> Indexed addressing (base register SI + offset)
  LDB  [BX+disp]          -> Based addressing (base register BX + displacement)
  ADD  Rd, Rs
  SUB  Rd, Rs
  STA  addr
  HLT
"""
 
 
class Simulator8085:
    def __init__(self, mem_size=64):
        self.reg = {"A": 0, "B": 0, "C": 0, "D": 0, "SI": 0, "BX": 0}
        self.memory = [0] * mem_size
        self.flags = {"Z": 0, "S": 0, "C": 0}
        self.pc = 0
 
    def set_flags(self, result):
        self.flags["Z"] = 1 if (result & 0xFF) == 0 else 0
        self.flags["S"] = 1 if (result & 0x80) else 0
        self.flags["C"] = 1 if result > 255 or result < 0 else 0
 
    def show(self, instr):
        print(f"\n{instr}")
        print("  Registers:", self.reg)
        print("  Flags    :", self.flags)
        watch = {10: self.memory[10], 15: self.memory[15], 20: self.memory[20],
                 30: self.memory[30], 40: self.memory[40]}
        print("  Memory (watched addresses):", watch)
 
    def run(self, program):
        self.pc = 0
        halted = False
        while self.pc < len(program) and not halted:
            line = program[self.pc]
            tokens = line.replace(",", " ").split()
            op = tokens[0].upper()
 
            if op == "MVI":                       # Immediate addressing
                reg, data = tokens[1], tokens[2]
                value = int(data.replace("#", ""))
                self.reg[reg] = value
 
            elif op == "LDA":                     # Direct addressing
                addr = int(tokens[1])
                self.reg["A"] = self.memory[addr]
 
            elif op == "STA":                     # Direct addressing (store)
                addr = int(tokens[1])
                self.memory[addr] = self.reg["A"]
 
            elif op == "LDAX":                    # Indirect addressing
                reg = tokens[1].strip("()")
                addr = self.reg[reg]
                self.reg["A"] = self.memory[addr]
 
            elif op == "MOV":                     # Register addressing
                rd, rs = tokens[1], tokens[2]
                self.reg[rd] = self.reg[rs]
 
            elif op == "LDX":                     # Indexed addressing [SI+offset]
                expr = tokens[1].strip("[]")
                base_reg, offset = expr.split("+")
                addr = self.reg[base_reg] + int(offset)
                self.reg["A"] = self.memory[addr]
 
            elif op == "LDB":                     # Based addressing [BX+disp]
                expr = tokens[1].strip("[]")
                base_reg, disp = expr.split("+")
                addr = self.reg[base_reg] + int(disp)
                self.reg["A"] = self.memory[addr]
 
            elif op == "ADD":
                rd, rs = tokens[1], tokens[2]
                result = self.reg[rd] + self.reg[rs]
                self.set_flags(result)
                self.reg[rd] = result & 0xFF
 
            elif op == "SUB":
                rd, rs = tokens[1], tokens[2]
                result = self.reg[rd] - self.reg[rs]
                self.set_flags(result)
                self.reg[rd] = result & 0xFF
 
            elif op == "HLT":
                halted = True
                self.show(line)
                break
 
            else:
                raise ValueError(f"Unknown instruction: {op}")
 
            self.show(line)
            self.pc += 1
 
        print("\nProgram halted." if halted else "\nProgram finished (ran off the end).")
 
 
if __name__ == "__main__":
    sim = Simulator8085()
    # pre-load some memory locations so LDA / LDAX / LDX / LDB have data to fetch
    sim.memory[10] = 25     # for direct addressing
    sim.memory[40] = 77     # for indirect addressing via register B
    sim.memory[15] = 9      # for indexed addressing via SI
    sim.memory[20] = 33     # for based addressing via BX
 
    program = [
        "MVI B, #40",        # immediate: B = 40 (used later as a pointer)
        "MVI SI, #10",       # immediate: SI = 10 (base for indexed mode)
        "MVI BX, #5",        # immediate: BX = 5  (base for based mode)
        "LDA 10",            # direct: A = MEM[10]  -> 25
        "STA 30",            # direct: MEM[30] = A
        "LDAX (B)",          # indirect: A = MEM[B] = MEM[40] -> 77
        "MOV C, A",          # register: C = A
        "LDX [SI+5]",        # indexed: A = MEM[SI+5] = MEM[15] -> 9
        "LDB [BX+15]",       # based:   A = MEM[BX+15] = MEM[20] -> 33
        "ADD C, A",          # C = C + A
        "SUB D, C",          # D = D - C  (D starts at 0, tests negative/flags)
        "HLT",
    ]
    sim.run(program)

Sample Output (key steps):

MVI B, #40
  Registers: {'A': 0, 'B': 40, 'C': 0, 'D': 0, 'SI': 0, 'BX': 0}
 
LDA 10
  Registers: {'A': 25, 'B': 40, 'C': 0, 'D': 0, 'SI': 0, 'BX': 0}
 
STA 30
  Memory (watched addresses): {10: 25, 15: 9, 20: 33, 30: 25, 40: 77}
 
LDAX (B)
  Registers: {'A': 77, 'B': 40, 'C': 0, 'D': 0, 'SI': 10, 'BX': 5}
 
LDX [SI+5]
  Registers: {'A': 9, 'B': 40, 'C': 77, 'D': 0, 'SI': 10, 'BX': 5}
 
LDB [BX+15]
  Registers: {'A': 33, 'B': 40, 'C': 77, 'D': 0, 'SI': 10, 'BX': 5}
 
SUB D, C
  Registers: {'A': 33, 'B': 40, 'C': 110, 'D': 146, 'SI': 10, 'BX': 5}
  Flags    : {'Z': 0, 'S': 1, 'C': 1}
 
Program halted.

## Observation

The watched memory addresses confirm each addressing mode reached the correct location: LDA 10 picks up the value 25 that direct addressing was supposed to fetch, LDAX (B) correctly follows register B as a pointer to fetch 77 from address 40, and both LDX and LDB correctly add their offset/displacement to the base register before reading memory. The final SUB instruction produces a negative logical result (D becomes 0 − 110), which the simulator represents using 8-bit wraparound, and this correctly sets both the Sign flag and the Carry flag, showing the flag logic is working as it would on real 8085 hardware.

# Question 5 – Number System Converter and Instruction Encoder/Decoder

Develop an application that converts binary, decimal, hexadecimal, and octal numbers, performs arithmetic operations in different number systems, and encodes simple assembly instructions into machine code based on predefined instruction formats and addressing modes. The program should also decode machine instructions back into assembly language.

## Approach

Conversion between any two of the four number systems is done in two steps: the input string is first converted to a plain Python integer using int(value, base), and that integer is then converted to the target base using Python's built-in bin(), oct() and hex() functions (with the '0b' / '0o' / '0x' prefixes stripped off). Routing every conversion through decimal in the middle keeps the code short and avoids writing sixteen separate conversion routines for every pair of bases.

Arithmetic directly in a base works the same way: both operands are converted to decimal, the addition or subtraction is carried out normally, and the result is converted back into the original base before being printed.

For the encoding part, a simple fixed 16-bit instruction format is used: 4 bits for the opcode, 4 bits for the destination register, 4 bits for the source register, and 4 bits for the addressing mode. Small lookup tables map instruction names, registers and addressing modes to their bit patterns, and the reverse lookup tables are used by the decoder to turn a 16-bit machine code string back into readable assembly, so the whole thing works as a matched encoder/decoder pair.

Source Code:

"""
Question 5 - Number System Converter and Instruction Encoder/Decoder
- Converts between binary, decimal, hexadecimal and octal.
- Performs arithmetic (+, -) directly in a chosen base.
- Encodes a simple assembly instruction into machine code using a
  fixed instruction format, and decodes machine code back to
  assembly.
"""
 
BASES = {"BIN": 2, "OCT": 8, "DEC": 10, "HEX": 16}
 
 
def to_decimal(value, base_name):
    return int(value, BASES[base_name])
 
 
def from_decimal(value, base_name):
    if base_name == "BIN":
        return bin(value)[2:] if value >= 0 else "-" + bin(-value)[3:]
    if base_name == "OCT":
        return oct(value)[2:] if value >= 0 else "-" + oct(-value)[3:]
    if base_name == "DEC":
        return str(value)
    if base_name == "HEX":
        return hex(value)[2:].upper() if value >= 0 else "-" + hex(-value)[3:].upper()
 
 
def convert(value, from_base, to_base):
    dec = to_decimal(value, from_base)
    return from_decimal(dec, to_base)
 
 
def arithmetic_in_base(a, b, base_name, op):
    da, db = to_decimal(a, base_name), to_decimal(b, base_name)
    result = da + db if op == "+" else da - db
    return from_decimal(result, base_name)
 
 
# ---------------- Instruction encoder / decoder ----------------
# Fixed 16-bit instruction format used for this exercise:
#   [ 4-bit opcode ][ 4-bit destination reg ][ 4-bit source reg ][ 4-bit addressing mode ]
OPCODES = {"MOV": "0001", "ADD": "0010", "SUB": "0011", "LOAD": "0100", "STORE": "0101"}
OPCODES_REV = {v: k for k, v in OPCODES.items()}
REGS = {"R0": "0000", "R1": "0001", "R2": "0010", "R3": "0011"}
REGS_REV = {v: k for k, v in REGS.items()}
MODES = {"REGISTER": "0000", "IMMEDIATE": "0001", "DIRECT": "0010", "INDIRECT": "0011"}
MODES_REV = {v: k for k, v in MODES.items()}
 
 
def encode_instruction(op, rd, rs, mode):
    return OPCODES[op] + REGS[rd] + REGS[rs] + MODES[mode]
 
 
def decode_instruction(code16):
    op_bits, rd_bits, rs_bits, mode_bits = code16[0:4], code16[4:8], code16[8:12], code16[12:16]
    return f"{OPCODES_REV[op_bits]} {REGS_REV[rd_bits]}, {REGS_REV[rs_bits]}   ; mode={MODES_REV[mode_bits]}"
 
 
if __name__ == "__main__":
    print("=== Number System Conversion ===")
    samples = [("101101", "BIN"), ("45", "DEC"), ("2F", "HEX"), ("57", "OCT")]
    for value, base in samples:
        print(f"{value} ({base}) -> "
              f"BIN:{convert(value, base, 'BIN')}  "
              f"OCT:{convert(value, base, 'OCT')}  "
              f"DEC:{convert(value, base, 'DEC')}  "
              f"HEX:{convert(value, base, 'HEX')}")
 
    print("\n=== Arithmetic directly in a given base ===")
    print("1010 (BIN) + 0110 (BIN) =", arithmetic_in_base("1010", "0110", "BIN", "+"))
    print("2F (HEX) + 11 (HEX)     =", arithmetic_in_base("2F", "11", "HEX", "+"))
    print("57 (OCT) - 17 (OCT)     =", arithmetic_in_base("57", "17", "OCT", "-"))
    print("120 (DEC) - 45 (DEC)    =", arithmetic_in_base("120", "45", "DEC", "-"))
 
    print("\n=== Instruction Encoding ===")
    instructions = [
        ("MOV", "R1", "R2", "REGISTER"),
        ("ADD", "R0", "R3", "REGISTER"),
        ("LOAD", "R2", "R1", "DIRECT"),
        ("STORE", "R3", "R0", "INDIRECT"),
    ]
    machine_codes = []
    for op, rd, rs, mode in instructions:
        code = encode_instruction(op, rd, rs, mode)
        machine_codes.append(code)
        print(f"{op} {rd}, {rs}  (mode={mode:9}) -> {code}  (hex: {from_decimal(int(code,2),'HEX').zfill(4)})")
 
    print("\n=== Instruction Decoding (round trip check) ===")
    for code in machine_codes:
        print(f"{code} -> {decode_instruction(code)}")

Sample Output:

=== Number System Conversion ===
101101 (BIN) -> BIN:101101  OCT:55  DEC:45  HEX:2D
45 (DEC) -> BIN:101101  OCT:55  DEC:45  HEX:2D
2F (HEX) -> BIN:101111  OCT:57  DEC:47  HEX:2F
57 (OCT) -> BIN:101111  OCT:57  DEC:47  HEX:2F
 
=== Arithmetic directly in a given base ===
1010 (BIN) + 0110 (BIN) = 10000
2F (HEX) + 11 (HEX)     = 40
57 (OCT) - 17 (OCT)     = 40
120 (DEC) - 45 (DEC)    = 75
 
=== Instruction Encoding ===
MOV R1, R2  (mode=REGISTER ) -> 0001000100100000  (hex: 1120)
ADD R0, R3  (mode=REGISTER ) -> 0010000000110000  (hex: 2030)
LOAD R2, R1  (mode=DIRECT   ) -> 0100001000010010  (hex: 4212)
STORE R3, R0  (mode=INDIRECT ) -> 0101001100000011  (hex: 5303)
 
=== Instruction Decoding (round trip check) ===
0001000100100000 -> MOV R1, R2   ; mode=REGISTER
0010000000110000 -> ADD R0, R3   ; mode=REGISTER
0100001000010010 -> LOAD R2, R1   ; mode=DIRECT
0101001100000011 -> STORE R3, R0   ; mode=INDIRECT

## Observation

The first block confirms that all four representations of the same underlying number line up correctly regardless of which base the value was originally entered in – for example 101101 in binary, 45 in decimal, and 2D in hexadecimal are all shown to be the same quantity. The arithmetic checks also come out correct: 1010 + 0110 in binary gives 10000, which is 10 + 6 = 16 in decimal, written back out in binary. The encode/decode round trip at the end shows that every instruction that was encoded into a 16-bit machine code string decodes back into exactly the original assembly instruction and addressing mode, which confirms the instruction format and the lookup tables are consistent with each other in both directions.